1 Make sure you have mastered the LED Blink exercise before starting this exercise.

2 Check the parts you we need to complete this project

.Parts

- Arduino

- 7x Leds of any kind (I use 5mm Red Leds)

- A 10k Resistor (brown black orange)

- 7x 220 or 330 Resistor (red red brown or orange orange brown)

- A little Push Button

- Breadboard

- Some wires for the breadboard

3 Build the LED dice on your breadboard using this circuit diagram:

4 Place the 7 LED’s in the pattern of an “H” as shown below:

5 Continue to build out the circuit until you use all the components

3 Now let populate the Sketch to drive the LED’s

int pinLeds1 = 10;

int pinLeds2 = 9;

int pinLeds3 = 7;

int pinLed4 = 8;

int buttonPin = 6;

int buttonState;

long ran;

int time = 2000;

void setup ()

{

pinMode (pinLeds1, OUTPUT);

pinMode (pinLeds2, OUTPUT);

pinMode (pinLeds3, OUTPUT);

pinMode (pinLed4, OUTPUT);

pinMode (buttonPin, INPUT);

randomSeed(analogRead(0));

}

void loop()

{

buttonState = digitalRead(buttonPin);

if (buttonState == HIGH){

ran = random(1, 7);

if (ran == 1){

digitalWrite (pinLed4, HIGH);

delay (time);

}

if (ran == 2){

digitalWrite (pinLeds1, HIGH);

delay (time);

}

if (ran == 3){

digitalWrite (pinLeds3, HIGH);

digitalWrite (pinLed4, HIGH);

delay (time);

}

if (ran == 4){

digitalWrite (pinLeds1, HIGH);

digitalWrite (pinLeds3, HIGH);

delay (time);

}

if (ran == 5){

digitalWrite (pinLeds1, HIGH);

digitalWrite (pinLeds3, HIGH);

digitalWrite (pinLed4, HIGH);

delay (time);

}

if (ran == 6){

digitalWrite (pinLeds1, HIGH);

digitalWrite (pinLeds2, HIGH);

digitalWrite (pinLeds3, HIGH);

delay (time);

}

}

digitalWrite (pinLeds1, LOW);

digitalWrite (pinLeds2, LOW);

digitalWrite (pinLeds3, LOW);

digitalWrite (pinLed4, LOW);

}

4 Now click the Upload icon to load the program to the Arduino. You will see the RX and TX lights on the Arduino flicker and a confirmation message. A few seconds later you should see your program in action!

What’s next?

Try changing

- The timing interval

- The Port number (don’t forget to rewire)

- Add another LED to you board and add code to make that blink

- Move on to the Dice Game!

For more information and additional lessons try the following useful links:

http://www.robotshop.com/blog/en/arduino-5-minute-tutorials-lesson-2-basic-code-blink-led-2-3639